Math Problems # 1

- You have 32.5 cubic meters of water needing to be moved.  How long will it take you to drain if you are pumping 75.6 Litres per minute?

- A Column full of water is 0.9144 meters in diameter and 4.572 meters high.

  - How many litres of water in the column?

  - If you drain 60% of the water at 89.54 Litres per minute, how long will it take you?

- What would the daily flow for a system by if you had recorded the following daily data:

Sunday – 14,356.25 Litres	Monday – 12,268.69 Litres

Tuesday – 16,895.46 Litres	Wednesday – 13,568.15 Litres

Thursday – 16,984.27 Litres	Friday – 12,864.86 Litres

Saturday – 15,968.33 Litres

- A tank is 15.6 meters long, 9.56 meters wide, and 6.39 meters deep.  What is the volume of the tank in litres?

- What is the draw down if the water level is 7.6 meters below ground when the pump is not in operation, and 13.05 meters when the pump is in operation?